| Introduction The goal of this activity is to find a way to predict the amount of time a generator will run on a given amount of fuel. The prediction will be based on data obtained experimentally. | WINCO Portable - Diesel; 1 Phase - 60 Hz 6 kW to 6.5 kW. The Winco Model W6010DEX is a Industrial Series generator with 6 kW (6 kVA) 60 Hz, of maximum output and 5.5 kW prime (continuous) power. Source: www.generatorjoe.net. |  |  |
| --- | --- | --- | --- |
| Directions - Consider the following scenario: Electric generators are used in a variety of applications. Often they provide power for signs or equipment when normal electrical service is not available. Construction companies will use an electric generator continuously to power their equipment. Generators are also found in homes and used in case of power failure. These units usually run on gasoline or diesel and have a relatively small tank in which the fuel is stored. A certain construction company has just purchased a new unit that holds 6.5 gallons of fuel and they want to collect some data to discover how long the fuel will last so they will know when to add fuel without continually checking the unit. They will also want to use this data to know how much fuel to purchase during a lunch break – to refuel the generator after lunch. (Refueling when the generator is cool is safest.) They experiment by putting exact quantities of fuel in an empty tank and measuring the length of time the generator runs before shutting off because the fuel is depleted. On the first day, they put in five and one-half gallons and the generator ran 4 hours and 30 minutes, then three gallons and the generator ran for 2 hour and 30 minutes. On the next day, they put in six and one-half gallons, lasting 5 hours and 30 minutes; then two gallons, lasting 1 hour and 40 minutes. On the third day, the generator ran 4 hours and 45 minutes on 5 and one-half gallons of fuel and then 2 hours and 30 minutes on three gallons of fuel. | The following data shows the typical wattage of equipment that might be used on a construction site for which power is supplied by a generator. Air Compressor, 1 HP 1500 W 3/8-inch 4A Drill 440 W 14-inch Band Saw 1100 W 7 1/4- inch Circular Saw 1400 W 9-inch Table Saw 1500 W 1500-watt Flood light 1500 W These figures estimate the amount of “running wattage” typical for this equipment. However, the “startup wattage” can be significantly more than the running wattage. Electrical equipment usually requires a surge in power to start the motor, only for a fraction of a second, and then the power required decreases and remains fairly constant. Startup wattage required Air Compressor, 1 HP 4500 W 3/8-inch 4A Drill 600 W 14-inch Band Saw 1400 W 7 1/4- inch Circular Saw 2300 W 9-inch Table Saw 3000 W 1500-watt Flood light 0 W |  |  |  |
| - What are the variables that need to be considered in this scenario? Which is the independent variable? Which is the dependent variable? |  |  |  |  |
| - Complete the table using the information provided in the scenario. | Amount of; Running. Fuel Used (gal.); Time (hours). 5.5. 3. 6.5; 5.5. 2. 5.5. 3. |  |
| - Construct a scatter plot of the data. |  |  |  |  |
| - Describe the trend you see in the data. Discuss the problem – the goal is to determine the amount of fuel a generator will need in order to run a predetermined amount of time. Do you think the data is sufficient? Reliable? In order to collect the data you need for a good model, would you have done anything different? |  |  |  |  |
| - Use the spreadsheet provided (2.3 Excel) to find an appropriate linear model for the data. |  |  |  |  |

| - Write the linear model you found in (6) using functional notation with the variables “fuel used” and “running time.” |  |
| --- | --- |
| - Predict the time that the generator will run on four gallons of fuel. |  |
| - How much fuel will be needed to run the generator ten hours? |  |
| - Solve the equation so that the construction crew can input the time they are going to want the generator to run making the output the number of gallons of fuel necessary. What is the slope of this new model? |  |
| - The generator that the crew purchased provides a maximum output of 6 kW. However, it is recommended that the generator not be required to sustain maximum output for no longer than 30 minutes. The manual says it will run well at 90% of the maximum output – called prime continuous power. Find the prime continuous power for this generator. If the crew actually needed a generator with 8kW of prime continuous power, what maximum output would they need to look for? |  |
| - The crew typically uses their 1 HP air compressor all day. The 3/8-inch drill, 14-inch band saw, 7 ¼-inch circular saw and the 9-inch table saw are used periodically. Sometimes, when working in dark areas, the crew uses several 1500-watt flood lights. What advice would you give the crew concerning what equipment to use and when? |  |

| - The starting wattage required by equipment is usually much greater than the power required to run it continuously. The graph shows the current needed to start a refrigerator motor. The refrigerator and most household appliances run on 120V current. The relationship between Amps, Watts, and Volts is Watts = Volts x Amps According to the graph, the refrigerator will require about 13A of starting current for a fraction of a second. That equates to 1560 W. 1560 Watts = 120 V x 13 A | http://www.generatorjoe.net/html/calculations.html |  |
| --- | --- | --- |
| - A graph showing the starting current required for a well pump is shown here. The motor is rated ¾ hp and 240V. Find the startup wattage required by the motor as well as the running wattage. | http://www.generatorjoe.net/html/calculations.html |  |